This is a README file for Wind Frequency Part of METCOMP

Version 2.0, Level 080110

1.  How to use the code

- Create a station/site ID list file.  The file consists of two columns for all stations: Column 1 is an integer station ID; Column 2 is a character string of station short abbreviation.  A sample file is “stn_id.dat” in ./test subdirectory

- Edit metcomp.inp file.  A sample file is in current directory.

- Prepare observation and prediction time series files.  The files MUST be paired for the observation and prediction, and the file names MUST be its base path/name plus the integer station IDs.  Refer to .\test directory for sample obs_xxxxx files and mm5_xxxxx files.

- Run METCOMP.EXE.   The frequency files will be named as their corresponding time series files except the extension is replace with “.frq”.

- The frq files can be used in the CALPRO GUI for windrose plots.

2.  Description of metcomp.inp

- Line 1: Title

- Line 2: METCOMP List file name

- Line 3: Base part of the FOUR output statistics file names. This base part will be added to the beginning of the four statistics files:

DailyTeHuStats.dat

DailyWindStats.dat

HourlyTeHuStats.dat

HourlyWindStats.dat

- Line 4: Station/Site ID file name. MAKE SURE no blank lines at the end of this file. This file consists of all stations/sites IDs.  Each station/site occupies one line with two columns:

Column 1: five- or six-digit station IDs. No blank space at beginning.

Column 2: a quoted string of station abbreviation. This column is

not used in METCOMP

- Line 5: Base name of input observation time series.

- Line 6: Base name of input prediction time series. Lines 4, 5, and 6 will be used in METCOMP to construct file names of input observation and prediction time series       extracted by METSERIES.  For examples, a line in Station ID file listed in Line 4 is

“71397  "Station1"”, Line 5 is “E:\test\obs_”, Line 6 is “F:\test\calmet_”, MTCOMP will look for two files:

E:\test\obs_71397.dat

F:\test\calmet_71397.dat

If these two files can not be found, METCOMP will be stopped.

- Line 7: Calm wind value. Wind speed equal or less than this value will be treated as calm wind.

- Line 8: Missing filter flag.  Setting to one (1) will remove hours with missing values from both observation and prediction time series before processing; Setting to zero (0) will keep hours with missing values in processing.

- Line 9: UTC time zone for processing. Currently it must be same as the UTC zone used in the time series.

- Line 10: Skip hours in observation and prediction time series. These two numbers can be useful when the beginning time in observations and predictions are not same.

3. Sample METCOMP.INP file with detailed description

MM5_EU Wind Speed and Direction Statistical Evaluation

metcomp.lst       : List file name

.\output\mtcp_    : Base part of statistics output files

.\test\stn_id.dat : Station IDs input file name

.\test\obs_    : Base name of input obs time series files

.\test\mm5_    : Base name of prediction timeseries files

0.999          : Calm wind threshold (m/s)

1              : Missing filter(1-Remove missing, 0-Keep missing)

0              : UTC base time zone (Must be same as in timeseries)

0,1            : Skip hours in OBS and prediction

--- LINES BELOW CONFIGURE WINDROSE OPTIONS --- (Do not delete this line)

4        : Number of time periods for additional analysis (NTPD)

00,06    : Time Range in the UTC above

06,12    : Time Range in the UTC above

12,18    : Time Range in the UTC above

18,00    : Time Range in the UTC above

0        : Number of WS ranges for additional analysis in m/s (NWSP)

****** Below are Comment lines *******************

Line 1: Title

Line 2: METCOMP List file name

Line 3: Base part of the FOUR output statistics file names. This base

part will be added to the beginning of the four statistics files:

DailyTeHuStats.dat

DailyWindStats.dat

HourlyTeHuStats.dat

HourlyWindStats.dat

Line 4: Station/Site ID file name. MAKE SURE no blank lines at the end

of this file. This file consists of all stations/sites IDs.

Each station/site occupies one line with two columns:

Column 1: five- or six-digit station IDs. No blank space at beginning.

Column 2: a quoted string of station abbreviation. This column is

not used in METCOMP

Line 5: Base name of input observation time series.

Line 6: Base name of input prediction  time series.

Lines 4, 5, and 6 will be used in METCOMP to construct file

names of input observation and prediction time series

extracted by METSERIES

Examples:

A line in Station ID file listed in Line 4 is

71397  "Station1"

Line 5 is E:\test\obs_

Line 6 is F:\test\calmet_

METCOMP will look for files:

E:\test\obs_71397.dat

F:\test\calmet_71397.dat

If these two files can not be found, METCOMP will be stopped.

Line 7: Calm wind value. Wind speed equal or less than this value

will be treated as calm wind.

Line 8: MIssing filter flag.  Setting to one (1) will remove hours with

missing values from both observation and prediction time series.

Setting to zero (0) will keep hours with missing values in

processing.

Line 9: UTC time zone for processing. Currently it must be same as

the UTC zone used in the time series.

Line 10: Skip hours in observation and prediction time series. These two

numbers can be useful when the beginning time in observation and

predition are not same.